HVAC PROPOSAL

X Mechanicals, LLC                                                07/23/25 
9815 Sam Furr Rd
Ste J140
Huntersville, NC  28078

Bird Dog Group
7804 Fairview Rd
Suite C-104
Charlotte, NC  28226

Ebert Residence
2440 Beretania Circle
Charlotte, NC

Thank you for the opportunity to provide HVAC for this project.  The HVAC Scope will be as follows.

EQUIPMENT

System 1 – Main First Floor (Gas/Air Conditioner)

Trane S9X2C100U5PSBB 95% 2 Stage 100,000 Btuh Gas Furnace
Trane 5TTR7048A1000A 4 Ton 17.0 SEER2 Air Conditioner
Trane 5TXCC007AS3HCA 4 Ton Evaporator Coil
Trane EFD210DLFR000A Trane CleanEffects® Whole Home Air Cleaner
Trane EERVR200A1P00B Energy Recovery Ventilator 200 CFM
1 - Honeywell T10 Pro WIFI Smart Thermostat
Indoor unit to be installed in Crawl Space

System 2 – Master/Study (Gas/Air Conditioner) w/ 2 ZONES (Master SuiteStudy)

Trane S9X2B060U4PSBB 95% 2 Stage 60,000 Btuh Gas Furnace
Trane 5TTR7024A1000A 2 Ton 17.0 SEER2 Air Conditioner
Trane 5TXCB006AS3HCA 2 Ton Evaporator Coil
Trane EFD175DLFR000A Trane CleanEffects® Whole Home Air Cleaner
Trane EERVR100A1P00B Energy Recovery Ventilator 100 CFM
2 - Honeywell T10 Pro WIFI Smart Thermostats
Honeywell Home Two Zone Zoning System
   Zone 1 – Master Suite
   Zone 2 – Study/Office
Indoor unit to be installed in Crawl Space

System 3 – Second Floor (Gas/Air Conditioner) w/ 4 ZONES (Second Floor Suites/Bonus)

Trane S9X2C100U5PSBB 95% 2 Stage 100,000 Btuh Gas Furnace
Trane 5TTR7048A1000A 4 Ton 17.0 SEER2 Air Conditioner
Trane 5TXCC007AS3HCA 4 Ton Evaporator Coil
Trane EFD210DLFR000A Trane CleanEffects® Whole Home Air Cleaner
Trane EERVR200A1P00B Energy Recovery Ventilator 200 CFM
4 - Honeywell T10 Pro WIFI Smart Thermostats Honeywell Home Two Zone Zoning System
   Zone 1 – Suite 3
   Zone 2 – Suite 4
   Zone 3 – Suite 5
   Zone 4 – Bonus Room
Indoor unit to be installed in Attic Space

System 4 – ADU (Gas/Air Conditioner) w/ 2 ZONES (First/Second Floors)

Trane S9X2B060U4PSBB 95% 2 Stage 60,000 Btuh Gas Furnace
Trane 5TTR7024A1000A 2 Ton 17.0 SEER2 Air Conditioner
Trane 5TXCB006AS3HCA 2 Ton Evaporator Coil
Trane EFD175DLFR000A Trane CleanEffects® Whole Home Air Cleaner
Trane EERVR100A1P00B Energy Recovery Ventilator 100 CFM
2 - Honeywell T10 Pro WIFI Smart Thermostats
Honeywell Home Two Zone Zoning System
   Zone 1 – First Floor
   Zone 2 – Second Floor
Indoor unit to be installed in Attic Space

EXHAUST FANS

- Master Water Closet His
Panasonic FV-08VRE2 80 CFM LoSone Exhaust Fan w Recessed Light (Fan Can)

- Master Water Closet Hers
Panasonic FV-08VRE2 80 CFM LoSone Exhaust Fan w Recessed Light (Fan Can)

- Master Bathroom
Panasonic FV-0510VSA1 100 CFM LoSone Exhaust Fan– Fan Only

- Office Powder
Panasonic FV-07VBB1 70 CFM LoSone Exhaust Fan– Fan Only

- First Floor Powder
Panasonic FV-08VRE2 80 CFM LoSone Exhaust Fan w Recessed Light (Fan Can)

- Guest Suite Bathroom
Panasonic FV-07VBB1 70 CFM LoSone Exhaust Fan– Fan Only

- 2nd Floor Powder
Panasonic FV-08VRE2 80 CFM LoSone Exhaust Fan w Recessed Light (Fan Can)

- Bath – Suite 3
Panasonic FV-07VBB1 70 CFM LoSone Exhaust Fan– Fan Only

- Bath – Suite 4
Panasonic FV-07VBB1 70 CFM LoSone Exhaust Fan– Fan Only

- Bath – Suite 5
Panasonic FV-07VBB1 70 CFM LoSone Exhaust Fan– Fan Only

- Studio Bathroom
Panasonic FV-07VBB1 70 CFM LoSone Exhaust Fan– Fan Only

- Wellness Bathroom
Panasonic FV-07VBB1 70 CFM LoSone Exhaust Fan– Fan Only

SCOPE OF WORK

(2) Dryer Vents Vented to Outside with Wall Dryer Box
(62) Supply Air Locations
(16) Return Air Locations
 (1) 10” Round Vent Connection for Range Vent - Indoor
       Includes connection provided Hood is on site at time of Trim Installation
(1) Broan MD10TU Make Up Air Exhauster with Switch – Indoor Range Hood Make Up Air
Ductwork to be R8 Duct board Plenums and R8 Flexible Duct.

Includes Labor, Materials, Equipment for a complete Turnkey job.
Rough In to be completed in Fifteen Days, Trim and Equipment Set (Outdoor) in Two Days; with Start Up on a separate day after meters are set.
Warranty will be Two Years from date of FINAL INSPECTION.

Gas Pipe (24 Openings) to:

- Indoor Main Fireplace

- Study Fire Place

- Outdoor Fireplace

- Future Outdoor Opening

- Courtyard Outdoor Opening

- Outdoor Kitchen

- Outdoor Grill

- 4 Gas Furnaces

- 3 Tankless Water Heaters – Located in Crawl

- 7 Gas Lanterns

- Future Generator

EXCLUSIONS

No Water Heater Vent/Flue materials included.  Water Heater vent installation is included, materials come with tankless water heaters which are proveided by Plumbing Contractor
No Fireplace Flue Included 
No Gas Pipe other than what is specifically noted above

BUDGET PRICE:                         $ 92,500.00

Thank you for the opportunity,

X Mechanicals, LLC

Bradley Heinrich
Founder & President

Email:  bheinrich@xmechanicals.com
Mobile:  (704) 996-5514
Service: (704) 975-6175
North Carolina License # 32593
South Carolina License # 38727
Mecklenburg County # X63382